EX NO: 1b: CHARACTERISTICS STUDY OF SIDE BAND POWER AND

FREQUENCIES IN AM USING MATLAB

Objectives:

i) To generate an Amplitude Modulated (AM) signal in MATLAB using specified carrier and

modulating signal parameters.

ii) To determine and verify the upper sideband (USB) and lower sideband (LSB) frequencies of

the AM signal using frequency spectrum analysis in MATLAB.

iii) To calculate the carrier power, sideband power, and total transmitted power of the AM signal

for a given modulation index using MATLAB.

iv) To analyze the effect of varying the modulation index on sideband power distribution and

overall transmission efficiency through MATLAB simulation.

PROGRAM (MATLAB CODE):

clc;

clear;

close all;

Am=1;

fm=500;

Ac=5;

fc=5000;

fs=20*fc;

t=0:1/fs:4/fm;

m = Am*sin(2*pi*fm*t);

c = Ac*sin(2*pi*fc*t);

mu = Am/Ac;

am = Ac*(1+mu*sin(2*pi*fm*t)).*sin(2*pi*fc*t);

dem = abs(hilbert(am)); dem = dem-mean(dem);

N=length(am); f=linspace(-fs/2,fs/2,N);

AM_FFT=abs(fftshift(fft(am)))/N;

USB=fc+fm; LSB=fc-fm;

Pc=Ac^2/2; Psb=(mu^2*Pc)/4;

figure('Name','AM Modulation','NumberTitle','off');

subplot(3,2,1), plot(t,m), title('Message'), grid on

subplot(3,2,2), plot(t,c), title('Carrier'), grid on

subplot(3,2,3), plot(t,am), title('AM Signal'), grid on

subplot(3,2,4), plot(t,dem), title('Demodulated'), grid on

subplot(3,2,5), plot(f,AM_FFT), xlim([fc-3*fm fc+3*fm]), title('Sidebands'), grid on

subplot(3,2,6), bar([Pc Psb Psb]), set(gca,'XTickLabel',{'Carrier','USB','LSB'}),

title('Power'), grid on

fprintf('μ=%.2f | USB=%d Hz | LSB=%d Hz | Pc=%.2f W | Psb=%.3f

W\n',mu,USB,LSB,Pc,Psb);

Output: